Engine (preliminary)

|  |  | SKYACTIV-G 1.5 | SKYACTIV-G 2.0 Standard Power | SKYACTIV-G 2.0 High Power with i-ELOOP | SKYACTIV-D 2.2 |
| --- | --- | --- | --- | --- | --- |
| Engines |  |  |  |  |  |
| Engine type |  | I4 DOHC 16 valves | I4 DOHC 16 valves | I4 DOHC 16 valves | I4 DOHC 16 valves |
| Displacement | cm3 | 1,496 | 1,998 | 1,998 | 2,184 |
| Bore x stroke | mm | 74.5x85.8 | 83.5x91.2 | 83.5x91.2 | 86.0x94.0 |
| Camshaft drive |  | Timing chain | Timing chain | Timing chain | Timing chain |
| Fuel injection system |  | Direct injection | Direct injection | Direct injection | Direct injection |
| Compression ratio |  | 14.0:1 | 14.0:1 | 14.0:1 | 14.0:1 |
| Emission control system |  | Three-way catalyst | Three -way catalyst | Three-way catalyst | Oxidation catalyst & DPF |
| Max. power | kW(PS)/ rpm | 74(100)/ 6,000 | 88(120)/6,000 | 121(165)/6,000 | 110(150)/4,500 |
| Max. torque | Nm/rpm | 150/4,000 | 210/4,000 | 210/4,000 | 380/1,800 |
| Fuel type |  | 95 RON | 95 RON | 95 RON | CN 51 (diesel) |
| Fuel tank capacity | l | 51 | 51 | 51 | 51 |
| Engine oil capacity | l | 3.9 | 4.0 | 4.0 | 4.8 |
| Battery |  | Q-85 | Q-85 | Q-85 | T-110 |
| Transmission |  | 6MT | 6MT/6AT | 6MT | 6MT/6AT |

SKYACTIV-MT transmission (preliminary)

|  | SKYACTIV-G 1.5 | SKYACTIV-G 2.0 Standard Power | SKYACTIV-G 2.0 High Power with i-ELOOP | SKYACTIV-D 2.2 |
| --- | --- | --- | --- | --- |
| Transmission type | 6-speed manual | 6-speed manual | 6-speed manual | 6-speed manual |
| Powertrain | FWD | FWD | FWD | FWD |
| Gear ratios |  |  |  |  |
| 1st | 3.700 | 3.700 | 3.363 | 3.357 |
| 2nd | 1.947 | 1.947 | 1.947 | 1.826 |
| 3rd | 1.300 | 1.300 | 1.300 | 1.565 |
| 4th | 1.029 | 1.029 | 1.029 | 1.085 |
| 5th | 0.837 | 0.837 | 0.837 | 0.854 |
| 6th | 0.680 | 0.680 | 0.680 | 0.711 |
| Reverse | 3.724 | 3.724 | 3.385 | 4.091 |
| Final drive ratio | 4.105 | 3.619 | 4.388 | 3.619 / 2.814 |

SKYACTIV-Drive transmission (preliminary)

|  | SKYACTIV-G 2.0 Standard Power | SKYACTIV-D 2.2 |
| --- | --- | --- |
| Transmission type | 6-speed automatic | 6-speed automatic |
| Powertrain | FWD | FWD |
| Gear ratios |  |  |
| 1st | 3.552 | 3.487 |
| 2nd | 2.022 | 1.992 |
| 3rd | 1.452 | 1.449 |
| 4th | 1.000 | 1.000 |
| 5th | 0.708 | 0.707 |
| 6th | 0.599 | 0.600 |
| Reverse | 3.893 | 3.990 |
| Final drive ratio | 4.325 | 3.548 |

Suspension and wheels (preliminary)

|  | SKYACTIV-G 1.5 | SKYACTIV-G 2.0 Standard Power | SKYACTIV-G 2.0 High Power | SKYACTIV-D 2.2 |
| --- | --- | --- | --- | --- |
| Suspension |  |  |  |  |
| Front suspension | MacPherson strut |  |  |  |
| Rear suspension | Multi-link |  |  |  |
| Stabilisers (front / rear) | 22.2 / 17 |  |  |  |
| Damper type (front & rear) | Twin tube |  |  |  |
| Wheels & tires |  |  |  |  |
| Wheel size | 16X6.5J 18X7J |  |  |  |
| Tire size | 205/60 R16 215/45 R18 |  |  |  |

Steering & brakes (preliminary)

|  |  | SKYACTIV-G 1.5 | SKYACTIV-G 2.0 Standard Power | SKYACTIV-G 2.0 High Power | SKYACTIV-D 2.2 |
| --- | --- | --- | --- | --- | --- |
| Steering |  |  |  |  |  |
| Steering type |  | Rack and pinion |  |  |  |
| Power assist type |  | Electric power assist |  |  |  |
| Steering gear ratio |  | 14.1 |  |  |  |
| Steering wheel turns (lock to lock) |  | 2.57 |  |  |  |
| Turning circle (kerb to kerb) | M | 5.3 |  |  |  |
| Turning circle (wall to wall) | M | 5.7 |  |  |  |
| Brakes |  |  |  |  |  |
| Type front |  | Ventilated discs |  |  |  |
| Type rear |  | Solid discs |  |  |  |
| Diameter front | mm | 280 | 295 | 295 | 295 |
| Diameter rear | mm | 265 | 265 | 265 | 265 |
| Vacuum booster diameter | Inches | 9 | 9 | 9 | 9 |
| Schedule maintenance |  | Every 20,000km / 12 months |  |  |  |

Performance and Weights (preliminary)

|  |  | SKYACTIV-G 1.5 | SKYACTIV-G 2.0 Standard Power |  | SKYACTIV-G 2.0 High Power with i-ELOOP | SKYACTIV-D 2.2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission |  | 6MT | 6MT | 6AT | 6MT | 6MT | 6AT |
| Performance |  |  |  |  |  |  |  |
| Top speed | km/h | 182 | 195 | 194 | 210 | 210 | 201 |
| Acceleration (0-100km/h)* | s | 10.8 | 8.9 | 10.4 | 8.2 | 8.1 | 9.7 |
| Emissions rating |  | Euro 5 | Euro 5 | Euro 5 | Euro 5 | Euro 6 | Euro 6 |
| Weight and payload |  |  |  |  |  |  |  |
| Min. kerb weight (without driver) | kg | 1,190 | 1,205 | 1,240 | 1,220 | 1,320 | 1,330 |
| Min. kerb weight ** | kg | 1,265 | 1,280 | 1,315 | 1,295 | 1,395 | 1,405 |
| Max. permissible weight | kg | 1,800 | 1,815 | 1,835 | 1,815 | 1,910 | 1,930 |
| Max. payload | kg | 610 | 610 | 595 | 595 | 590 | 600 |
| Permissible front axle weight | kg | 930 | 960 | 980 | 960 | 1,060 | 1,075 |
| Permissible rear axle weight | kg | 945 | 930 | 930 | 930 | 925 | 930 |
| Permissible tow weight, trailer without brakes | kg | 600 | 600 | 600 | 600 | 650 | 650 |
| Permissible tow weight, trailer with brakes (12% slope) | kg | 950 | 1,300 | 1,300 | 1,300 | 1,500 | 1,500 |
| Permissible tow weight, trailer with brakes (8% slope) | kg | 1,500 | 1,600 | 1,600 | 1,600 | 1,800 | 1,500 |
| Max. roof load capacity | kg | 75 | 75 | 75 | 75 | 75 | 75 |

* Under Mazda test conditions

** Incl. 75kg drive